Upgrade Procedures

- Enter into upgrade mode.

  - Open upgrade tool “GD32 MCU IAP Tool.exe”, connect BD200 and PC, if the upgrade tool recognizes BD200, it will display as below. And BD200 enters into upgrade mode.

  - If the upgrade tool does not recognize BD200, BD200 will not enter into upgrade mode.

Disconnect the USB cable.

Press and hold the dial when connect USB cable to BD200, and keep holding the dial for 10 seconds.

BD200 will enter into upgrade mode.

- Upload ‘.hex’ file.

  - Click ‘open’ in the ‘Download APP program file’ chart. And find “USBD_main_discharge.hex” file.

- Upgrade

  - After the ‘.hex’ file is loaded, click ‘OK’ to start upgrade.

  - NEVER DISCONNECT USB CONNECTION OR INTERRUPT THE UPGRADE PROCESS, OTHERWISE MAY CAUSE DAMAGE TO DEVICE.

  - Upgrade in process.

  - When the process finishes successfully, you will see below message chart.

- Disconnect the USB cable. Connect battery to BD200 and power on. Then connect BD200 to PC again, you can use PC software.